START_HERE – Smart Plant Guardian

Welcome to the Smart Plant Guardian project! This guide gives you a step-by-step onboarding to install, run, and understand the full system.

(1) Overview

Smart Plant Guardian is a real-time IoT plant-health monitoring system built using:

🌱 Arduino Nicla Sense ME – captures environmental data

📡 Arduino MKR WiFi 1010 – sends sensor readings wirelessly (MQTT)

💾 SQLite Database – stores clean, timestamped readings w/ metadata

📊 Python Dash Dashboard – visualises live plant conditions

You’ll monitor:

• Temperature

• Humidity

• VOC Levels

• Air Pressure

• Plant Health Score

(2) What You Need

Hardware

Arduino Nicla Sense ME / Arduino MKR WiFi 1010

USB-C / Micro-USB cables for Battery Pack

Peace Lily (recommended plant)

Laptop

(3) Software

Python 3.10+

Git

pip or Conda

Arduino IDE

VS Code (optional)

(4) Clone the Repository

Open Terminal / CMD and run:

git clone https://github.com/HHPatel1/SmartPlantGuardian

(5) Create and Activate the Python Environment

Mac / Linux:

python3 -m venv .venv

source .venv/bin/activate

Windows:

python -m venv .venv

.venv\Scripts\activate

Install dependencies:

pip install -r requirements.txt

(6) Create Your .env File

Inside the repo root, create a file called .env with:

MQTT_BROKER=test.mosquitto.org

MQTT_PORT=1883

SERIAL_PORT=/dev/tty.usbmodemxxxx

EMAIL_USER=your_email@example.com

EMAIL_PASS=your_password

(7) Initialise the Database

Run:

python src/utils/db_helpers.py

This creates:

data/database/plant_data.sqlite

(8) Choose Your Data Acquisition Mode

MQTT Wireless Mode

Upload WiFi firmware

Upload:

src/acquisition/arduino/mkr_wifi_plant_mqtt.ino

Start the subscriber:

python src/acquisition/mqtt_subscriber.py

(9) Launch the Dashboard

Run:

python src/dashboard/dashboard.py

Go to:

📍 http://127.0.0.1:8050/

Dashboard contains:

4x live graphs

Plant health score w/ colour indicator

A plant photo

Email alert tab

(10) Repository Map (How Everything Fits Together)

src/                → All Python source code

src/dashboard/      → Dashboard UI components

src/acquisition/    → MQTT + Serial acquisition scripts

data/raw/           → Raw CSV logs

data/database/      → SQLite database

metadata/           → Calibration + plant profile + experiment info

docs/               → Project overview + screenshots + diagrams

results/            → Saved plots + reports